Beechcraft Super King Air Model 300

REFERENCE INFORMATION

| Total Flight Simulator aircraft weight with full fuel | 14,100 lbs |
| --- | --- |
| Empty weight with 0 fuel | 8,881 lbs |

| VMO - Maximum Operating Speed | 259 KIAS |
| --- | --- |
| MMO - Maximum Operating Speed Mach | 0.58 Mach |
| Turbulent Air Penetration Speed | 170 KIAS |
| VLE - Maximum Gear Operating Speed | 181 KIAS |
| VLO - Maximum Gear Retraction Speed | 163 KIAS |
| VA - Maneuvering Speed | 200 KIAS |
| VMC - Minimum Control Speed | 91 KIAS |
| VX - Two-engine Best Angle-of-Climb | - |
| VY - Two-engine Best Rate of Climb | - |
| Best Cruise Altitude non-RVSM | FL260 - FL280 |

Maximum Flap Placard Speeds

| Flaps | KIAS |
| --- | --- |
| Approach | 200 |
| Full extension | 157 |

V1 - Takeoff Decision Speed dry runway; 
flaps up; 10˚C, 6,000' pressure altitude

| 14,000 lbs | 116 KIAS |
| --- | --- |
| <10,500 lbs | 110 KIAS |

VR - Rotation Speed dry runway, flaps up

10˚C, 6,000' pressure altitude

| 14,000 lbs | 116 KIAS |
| --- | --- |
| <10,500 lbs | 110 KIAS |

V2 - Takeoff Safety Speed dry runway, flaps up

10˚C, 6,000' pressure altitude

| 14,000 lbs | 117 KIAS |
| --- | --- |
| <10,500 lbs | 111 KIAS |

VYSE - Single Engine Best Rate of Climb

flaps up;  10˚C, 6,000' pressure altitude

| 14,000 lbs | 117 KIAS |
| --- | --- |
| 12,000 lbs | 113 KIAS |
| <10,500 lbs | 109 KIAS |

Best Glide Speed, flaps up

| 14,000 lbs | 113 KIAS |
| --- | --- |
| 12,000 lbs | 109 KIAS |
| <10,500 lbs | 100 KIAS |

V50 - Speed after reaching 500' flaps up

10˚C, 6,000' pressure altitude

| 14,000 lbs | 127 KIAS |
| --- | --- |
| <10,500 lbs | 121 KIAS |

Cruise Climb Speeds

| Sea level to 10,000 | 160 KIAS |
| --- | --- |
| 10,000 to 20,000 | 140 KIAS |
| 20,000 to 25,000 | 130 KIAS |
| 25,000 to 30,000 | 120 KIAS |

Approach Speeds

| Downwind | 160 KIAS |
| --- | --- |
| Base | 140 KIAS |
| Final Approach | VREF + 10 KIAS + 1/2 Gust Spd |

VREF - Landing Touchdown Speed

Flaps 100%; gear down

Do not interpolate, use the next higher up speed

| 14,000 lbs | 104 KIAS |
| --- | --- |
| 13,500 lbs | 99 KIAS |
| 12,000 lbs | 99 KIAS |
| 11,000 lbs | 99 KIAS |
| 10,000 lbs | 99 KIAS |
| 9,000 lbs | 99 KIAS |

VREF - Landing Touchdown Speed

Flaps 40%; gear down

Do not interpolate, use the next higher up speed

| 14,000 lbs | 114 KIAS |
| --- | --- |
| 13,500 lbs | 113 KIAS |
| 13,000 lbs | 112 KIAS |
| 11,000 lbs | 105 KIAS |
| 9,000 lbs | 104 KIAS |

VREF - Landing Touchdown Speed

Flaps UP; gear down

Do not interpolate, use the next higher up speed

| 14,000 lbs | 124 KIAS |
| --- | --- |
| 13,500 lbs | 123 KIAS |
| 13,000 lbs | 122 KIAS |
| 11,000 lbs | 115 KIAS |
| 9,000 lbs | 104 KIAS |

To... 					Press...

Display/hide Main panel	SHIFT+1

Display/hide Radio panel	SHIFT+2

Display/hide GPS	SHIFT+3

Display/hide Engine controls	SHIFT+4

Display/hide Caution Annunciator panel	SHIFT+5

Display/hide Warning Annunciator panel	SHIFT+6

Display/hide Compass	SHIFT+7

Display/hide Trim	SHIFT+8

Display/hide Fuel panel	SHIFT+9

BEFORE ENGINE STARTING

[ ] Cabin door	LOCKED

[ ] Weight Limits	CHECKED

[ ] Parking Brake	SET
(press CTRL+PERIOD key)

[ ] Landing Gear	CHECK DOWN

[ ] Power Levers	IDLE
(press F2 until Idle)

[ ] Propeller Controls	FULL FORWARD
(press CTRL+F3 until fully in)

[ ] Condition Levers	FUEL CUTOFF
(press CTRL+SHIFT+F2 until fully out)

[ ] Fasten Seatbelts Light	ON

[ ] Cabin Temp Mode	OFF

[ ] Vent Blower	AUTO

[ ] Aft Blower	OFF

[ ] Electrical Heaters	OFF

[ ] Battery Switch	ON

[ ] Annunciator Lights	TEST

[ ] Oxygen Supply Pressure	CHECK

[ ] Standby Pumps	OFF

[ ] Fuel Pressure Warning	ILLUMINATED

[ ] Fuel Quantity	CHECK (Main and Auxiliary)

[ ] DC Voltmeter	CHECK

[ ] Rotating Beacon	ON

ENGINE STARTING (BATTERY)

Press CTRL+E to initiate engine autostart sequence, or:

[ ] Battery Switch	ON

[ ] Beacon	ON

[ ] Propeller Area	CLEAR

[ ] Right Engine Start Switch	ON
(Click and hold until engine starts)

[ ] Right Stable N1 > 12%	WAIT

[ ] Right Condition Lever	LOW IDLE
(after the N1 Turbine RPM has stabilized, 12% minimum)

[ ] R. ITT, N1 Turbine RPM	MONITOR
(1000 ˚C max. rise in 10 seconds)

[ ] "R FUEL PRESS LO"	EXTINGUISHED

Annunciator

[ ] "R OIL PRESS LO"	EXTINGUISHED

Annunciator

[ ] Right Oil Pressure	CHECK

[ ] Right Condition Lever	HIGH IDLE

[ ] --- WAIT ---	N1 = 50%

[ ] Right Engine Start Switch	OFF

[ ] Right Generator	ON

ENGINE STARTING (BATTERY) cont.

[ ] Charge Battery	LOAD = 0.5, MAX. 5 MINUTES

[ ] Right Generator	OFF

[ ] Left Engine Start Switch	ON
(Click and hold until engine starts)

[ ] Left Stable N1 > 12%	WAIT

[ ] Left Condition Lever	LOW IDLE
(after the N1 Turbine RPM has stabilized, 12% minimum)

[ ] Right Generator	ON

[ ] L. ITT & N1 Turbine RPM	MONITOR

[ ] "L FUEL PRESS LO"	EXTINGUISHED

Annunciator

[ ] "L OIL PRESS LO"	EXTINGUISHED

Annunciator

[ ] Left Oil Pressure	CHECK

[ ] --- WAIT ---	N1 = 50%

[ ] Left Engine Start Switch	OFF

[ ] Left Generator	ON

[ ] Right Condition Lever	REDUCE TO LOW IDLE

AFTER START

[ ] Inverters	CHECK

[ ] DC Voltage/Load	CHECK

[ ] Avionics Master Switch	ON

[ ] Lights	AS REQUIRED

[ ] Cabin Temp/Mode	AS REQUIRED

[ ] Annunciators	TEST, CLEAR

[ ] Fuel Quantity, Flight,	CHECK

& Engine Instruments

TAXI

[ ] Brakes	CHECK

[ ] Gyros	CHECK

BEFORE TAKEOFF (RUNUP)

[ ] Avionics	CHECK

[ ] Pressurization	CHECK, SET
(alt. + 500 (cabin 500 agl, rate)

[ ] Electric Pitch Trim Ctrl	CHECK

[ ] Trim Tabs	SET
(with Num Lock OFF press 
Num Pad 1 or Num Pad 7 as necessary)

[ ] Flaps	CHECK, SET

[ ] Flight Controls	FULL, FREE

[ ] Propellers	FULL FORWARD

[ ] Condition Levers	FULL FORWARD

[ ] Primary Governors	EXERCISE AT 1500 RPM

[ ] Bleed Air Valves	PNEU & ENVIR OFF

[ ] Pneumatic Gauge	0 PSI

[ ] "BLEED AIR FAIL"	BOTH ILLUMINATED

Annunciators

[ ] Bleed Air Valves	ENVIR OFF or OPEN

[ ] Pneumatic Gauge	GREEN ARC

[ ] "BLEED AIR FAIL"	BOTH EXTINGUISHED

Annunciators

[ ] Engine Anti-Ice	ON

[ ] Torque Drop	MONITOR

[ ] Engine Anti-Ice	OFF

[ ] Torque Returns	MONITOR

[ ] Autofeather	ARM

[ ] Manual Prop. Feathering	CHECK

[ ] Anti-ice	AS REQUIRED

[ ] Fuel Quantity, Flight,	CHECK

& Engine Instruments

BEFORE TAKEOFF (FINAL ITEMS)

[ ] Annunciator Lights	EXTINGUISHED OR CONSIDERED

[ ] Transponder	ON ALT

[ ] Lights	AS REQUIRED

[ ] Anti-ice	AS REQUIRED

[ ] Bleed Air Valves	OPEN

[ ] Takeoff speeds	CONFIRM

(V1, VR, & V2)

TAKEOFF

[ ] Brakes	HOLD

[ ] Power Levers	100% N1 TURBINE RPM
(OBSERVE ITT LIMITS)
(press F3 as necessary)

[ ] Brakes	RELEASE
(press PERIOD key)

[ ] VR	ROTATE TO APPROX. 10 DEGREES PITCH UP

[ ] Landing Gear	RETRACT (WHEN POSITIVE CLIMB ESTABLISHED)
(press G)

[ ] Airspeed	MAINTAIN V2 UNTIL CLEAR OF OBSTACLES

[ ] Flaps	RETRACT FULLY (BLUE LINE, 125 KIAS MINIMUM)

AFTER TAKEOFF/CLIMB

[ ] Landing Gear	CHECK RETRACTED

[ ] Flaps	CHECK RETRACTED

[ ] Yaw Damper	ON

[ ] Climb Power	90% N1 TURBINE RPM (OR AS DESIRED)
(press F2 or F3 as necessary)

[ ] Propellers	1700 RPM (OR AS DESIRED)
(press CTRL+F2 or CTRL+F3 as necessary)

[ ] Propeller Synchrophaser	ON

[ ] Autofeather	OFF

[ ] Auto-Ignition	AS REQUIRED

[ ] Fasten Seatbelts Light	AS REQUIRED

[ ] Cabin Pressure	AS REQUIRED

[ ] Aft Blower	OFF

[ ] Engine Instruments	MONITOR

[ ] Lights	AS REQUIRED

CRUISE (Best Altitude FL260 - FL280 Non-RVSM)

[ ] Propellers	1500 RPM (OR AS DESIRED
ITT to 800, not to exceed 91 Ft/lbs torque or 104% N1
(press CTRL+F2 or CTRL+F3 as necessary)

[ ] Cruise Power	AS DESIRED
~Fuel Flow = 300 lbs/min R/L
(press F2 or F3 as necessary)

[ ] Engine Instruments	MONITOR

[ ] Autofeather	OFF

DESCENT

[ ] Pressurization	SET CABIN ALTITUDE, RATE

[ ] Altimeter	SET

[ ] Fasten Seatbelts Light	AS REQUIRED

[ ] Windshield Anti-Ice	AS REQUIRED

[ ] Autofeather	ARM

[ ] Fuel Balance	CHECK

[ ] Power	AS REQUIRED
N1 NEVER LESS THAN 65%
(press F2 or F3 as necessary)

BEFORE LANDING

[ ] Pressurization Light	CHECK

[ ] Fasten Seatbelts Light	ON

[ ] Autofeather	VERIFY ARMED

[ ] Prop Synchrophaser	AS REQUIRED

[ ] Ice Protection	AS REQUIRED

[ ] Auto-Ignition	ARMED

[ ] VREF	CONFIRM

[ ] Altimeter	SET

[ ] Flaps	APPROACH
(press F7 until APPR)

[ ] Landing Gear	DOWN
(press G)

[ ] Landing Lights	AS REQUIRED

ON FINAL

[ ]Flaps	DOWN

[ ]Yaw Damper	OFF

[ ]Propeller Levers	HIGH RPM

BALKED LANDING

[ ] Power Levers	MAXIMUM
(press F3 until fully in)

[ ] Propeller Levers	FULL FORWARD
(press CTRL+F3 until fully in)

[ ] Airspeed	100 KTS UNTIL CLEAR OF OBSTACLES

[ ] Flaps	UP
(press F5 key)

[ ] Landing Gear	RETRACT
(press G key)

NORMAL LANDING

--When landing is assured--

[ ] Flaps	100%
(press F7 until DOWN)

[ ] Airspeed	VREF

[ ] Condition Levers	HIGH IDLE
(press CTRL+SHIFT+F3 until fully in)

[ ] Propeller Levers	FULL FORWARD
(press CTRL+F3 until fully in)

[ ] Power Levers	IDLE
(press F2 until Idle)

--After touchdown--

[ ] Power Levers	FULL REVERSE
(press F2 until fully out)

--Below 60 KTS--

[ ] Brakes	AS REQUIRED
(press PERIOD key)

[ ] Power Levers	GROUND FINE
(press F3 until fully out)

AFTER LANDING

[ ] Landing & Taxi Lights	AS REQUIRED

[ ] Engine Auto-Ignition	OFF

[ ] Anti-ice	OFF

[ ] Interior Lights	AS REQUIRED

[ ] Electrics	OBSERVE LOAD LIMITS

[ ] Transponder	STANDBY
(press SHIFT+2 to display radio stack)

[ ] Trim Tabs	SET
(with Num Lock off press Num Pad 1 or Num Pad 7 as necessary)

[ ] Flaps	UP
(press F6 until UP)

[ ] Strobes	OFF

SHUTDOWN AND SECURING

[ ] Parking Brake	SET
(press CTRL+PERIOD key)

[ ] Inverters	OFF

[ ] Avionics Master	OFF

[ ] Autofeather	OFF

[ ] Lights	OFF

[ ] Cabin Temp Mode	OFF

[ ] Vent Blower	OFF

[ ] Aft Blower	OFF

[ ] Battery	CHARGED

[ ] ITT	STABILIZED AT MINIMUM TEMPERATURE FOR 1 MINUTE

[ ] Condition Levers	FUEL CUTOFF
(press CTRL+SHIFT+F2 until fully out)

[ ] Propellers	FEATHER
(press CTRL+F2 until fully out)

[ ] Standby Pumps, Crossfeed	OFF

[ ] DC Volt	CHECK VOLTAGE/LOAD

[ ] Overhead Panel Switches	OFF

[ ] Batt. and Gen. Switches	OFF (below 15% N1)

[ ] Wheel Chock	CONFIRM INSTALLED

[ ] External Covers	CONFIRM INSTALLED

ABNORMAL CHECKLISTS

AIR START

[ ] Cabin Temp	OFF

[ ] Vent Blower	AUTO

[ ] Aft Blower	OFF

[ ] Radiant Heat	OFF

[ ] Windshield Heat	OFF

[ ] Generator	OFF

[ ] Power Lever	IDLE
(press F2 until fully out)

[ ] Condition Levers	FUEL CUTOFF
(press CTRL+SHIFT+F2 until fully out)

[ ] Ignition/Engine Start 	ON

[ ] Condition Levers	LOW IDLE
(press CTRL+SHIFT+F3 until levers are at mid range)

[ ] N1 > 50%	--- WAIT ---

[ ] Ignition/Engine Start 	OFF

[ ] Propeller Levers 	AS REQUIRED
(press CTRL+F2 / CTRL+F3 as desired)

[ ] Power Levers 	AS REQUIRED
(press F2/F3 as desired)

[ ] Generators	ON

[ ] Engine Auto Ignition Switch	ARM

[ ] Electrical Equipment	AS REQUIRED

ENGINE FAILURE BEFORE V1

[ ] Power Levers	GROUND FINE

[ ] Brakes	AS REQUIRED
(press PERIOD key)

[ ] Operative Engine	MAX REVERSE
(Watch Traction!)

IF INSUFFICIENT RUNWAY FOR STOPPING

[ ] Condition Levers	FUEL CUTOFF
(press CTRL+SHIFT+F2 until fully out)

[ ] Fire Pull Handles	PULL

[ ] Master Switch	OFF WITH GANG BAR

ENGINE FAILURE AFTER LIFT-OFF (V1)

(Can't land on remaining runway)

[ ] Power Levers	MAXIMUM
(press F3 until fully in)

[ ] Propeller Levers	FULL FORWARD
(press CTRL+F3 until fully in)

[ ] Airspeed	MAINTAIN
(Takeoff Speed or Above)

[ ] Pitch After Rotation	10˚

[ ] Climb Airspeed	V2, POSITIVE CLIMB INDICATIONS

[ ] Landing Gear	 RETRACTED
(press G key)

[ ] Power Lever (Inop. Eng.)	IDLE AFTER AUTOFEATHER

[ ] Propeller Lever (Inop. Eng).	FEATHER

[ ] Airspeed	VYSE
(After Obstacles Cleared)

[ ] 500 ft AGL	LEVEL OFF AND ACCEL. to VENR

[ ] Flaps	UP AFTER VENR
(press F5 key)

[ ] Landing Lights	OFF

[ ] Engine Cleanup	PERFORM

[ ] Operating Engine	 REDUCE POWER TO MAXIMUM CONTINUOUS

[ ] 	CONTINUE CLIMB AT VENR

[ ] Land	AS SOON AS PRACTICABLE

NOTE: Holding 5 degrees of bank towards the operating engine will assist in maintaining directional control and will improve aircraft performance.

Clean-up Inoperative Engine

[ ] Condition Lever	FUEL CUTOFF
(press CTRL+SHIFT+F2 until fully out)

[ ] Bleed Air Valve	AS REQUIRED

[ ] Engine Auto Ignition Switch	OFF

[ ] Autofeather Switch	OFF

[ ] Generator	OFF

[ ] Synchrophaser	OFF

[ ] Electrical Load	MONITOR

ENGINE MALFUNCTION DURING FLIGHT

[ ] Autopilot/Yaw Damp	DISENGAGE
(press Z Key if AP is on)

[ ] Power	 AS REQUIRED
(press F3 until fully in)

[ ] Inoperative Engine 	IDENTIFY

[ ] Power Lever (Inop. Eng.)	FUEL CUTOFF

[ ] Propeller Lever (Inop. Eng.)	VERIFY FEATHERED

[ ] Condition Lever (Inop. Eng.)	FUEL CUTOFF

[ ] Landing Gear	AS REQUIRED

[ ] Flaps	AS REQUIRED

[ ] Engine Cleanup	PERFORM

[ ] Power Lever (Oper. Eng.)	SINGLE ENGINE CRUISE

[ ] Land	AS SOON AS PRACTICABLE

2nd ENGINE FLAME-OUT

[ ] Power Levers	IDLE
(press F2 until fully in)

[ ] Propeller Levers	DO NOT FEATHER

[ ] Condition Levers	FUEL CUTOFF
(press CTRL+SHIFT+F2 until fully out)

[ ] 	CONDUCT AIR START

GLIDE

[ ] Landing Gear	RETRACTED
(press G key if gear is down)

[ ] Flaps	 RETRACTED
(press F5 key)

TRY RESTART BEFORE FEATHER BOTH PROPELLERS

[ ] Propellers	FEATHERED
(press CTRL+F2 until fully out)

[ ] Airspeed	135 KIAS

EMERGENCY DESCENT

[ ] Power Levers	MAXIMUM
(press F3 until fully in)

[ ] Propeller Levers	FULL FORWARD
(press CTRL+F3 until fully in)

[ ] Flaps	APPROACH
(press F7 key from flaps up)

[ ] Landing Gear	RETRACTED
(press G key)

[ ] Airspeed	181 KIAS (VLE)

PRESSURIZATION DIFFERENTIAL IN RED

[ ] Cabin Altitude Selector	SELECT HIGHER ALTITUDE

IF CONDITION PERSISTS:

[ ] Bleed Air Valves	ENVIR OFF

[ ] Cabin Pressure Switch	DUMP

[ ] Bleed Air Valves	OPEN